| 1. GENERAL INFORMATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name of project |  |  |  |  |  |
| Name of person responsible |  |  |  |  |  |
| Address |  |  |  |  |  |
| Phone number |  |  |  |  |  |
| Municipality where project is located |  |  |  |  |  |
| Attach a general location map |  |  |  |  |  |
| Have you contacted the municipality | YES NO |  | NOTE: MUNICIPAL PERMITS MAY BE REQUIRED |  |  |
| Have any naturally occurring geologic formations or soil conditions that may have the potential to cause pollution during earth disturbance activities been identified onsite? YES NO If yes, explain: |  |  |  |  |  |
| Brief description of project |  |  |  |  |  |
| Estimated start and end dates | Start Date | 7/15/15 |  | End date | 7/17/15 |
| Name of nearest receiving stream | Bird Run |  |  | Distance | Feet |

Erosion and Sedimentation Pollution Control (ESPC) Plan Narrative

| 2. SOILS INFORMATION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |
| SOIL NAME | SLOPE |  | DEPTH |  | IS SOIL HYDRIC | YES NO |
| LIMITATIONS |  |  |  |  |  |  |

Note 1 – Soils can be found at the Web Soil Survey ( http://websoilsurvey.nrcs.usda.gov/app/)

Note 2 - Hydric soils may indicate the presence of wetlands and that these areas should be avoided.  If not, permits from PADEP and/or US Army Corps of Engineers may be required

3. Typical Construction Sequence

In order for an ESPC plan to be effective, construction must take place in an organized sequence that integrates the ESPC Best Management Practices (BMPs) into the sequence in a timely manner. The first step in nearly all projects is the installation of sediment barriers below the project, that is downslope and installation of BMPs to control off site runoff that drains to the site. The sequence would then include other needed BMPs and end with the removal of temporary controls. A typical construction sequence is provided below.

A. Contact your local Municipality and obtain any necessary permits

B. Select appropriate area for burial (see regulations and the cover sheet attached for minimum standard and setback requirements)

C. Install perimeter controls (Silt fence, compost filter sock, straw bales, rock construction entrance, diversion swale)

D. Excavate burial area

E. Backfill burial area with a minimum of 2 feet of soil within 48 hours

F. Seed and Mulch all disturbed areas according to DEP’s Erosion Control Manual

G. Ensure all disturbed areas achieve a minimum 70% uniform vegetative cover

| 4. TEMPORARY CONTROLS |  |  |  |
| --- | --- | --- | --- |
| Check the temporary controls that will be implemented for your project. Attach a drawing or design for any practice that is not listed in this guide. |  |  |  |
| Vegetated stabilization |  | Vegetated swale |  |
| Vegetated filter strip |  | Rock lined Swale |  |
| Silt fence |  | Rock filter |  |
| Filter sock |  | Rock outlet protection |  |
| Straw bales |  | Sediment trap |  |
| Rock construction entrance |  | Inlet filter bag |  |
| Other |  |  |  |
| Other |  |  |  |
| Other |  |  |  |

| 5. THERMAL IMPACTS |  |  |  |
| --- | --- | --- | --- |
| Identify BMPs to avoid, minimize or mitigate potential pollution from thermal impacts: |  |  |  |
| Vegetation/Vegetated Filter Strip |  | Locate project away from Waters of the Commonwealth |  |
| Limit Extent / Duration of Earth Disturbance |  | Avoid direct discharge to Waters of the Commonwealth |  |
| Other: |  | Other: |  |

6. PERMANENT CONTROLS

When a project is completed, it is necessary to ensure permanent stabilization of the site. This entails the stabilization of all exposed soil areas through revegetation, stone, pavement or other stabilization practice. List the permanent stabilization BMPs to be used below. Permanent stabilization BMPs should follow appropriate recommendations.

| 6. MAINTENANCE PROGRAM |  |
| --- | --- |
| All ESPC BMPs must be properly maintained in order to ensure that they function properly. List maintenance measures to be implemented below. The first four are general measures and are required. Common maintenance practices are also listed. Check those that apply. For maintenance practices not listed, describe the practice in the blank lines below and check. |  |
| Until final site stabilization, all ESPC BMPs will be properly inspected weekly and after each rainfall | X |
| All maintenance for ESPC BMPs will be conducted according to the recommendations in this guide | X |
| If any ESPC practice proves to be inadequate, additional measures will be immediately implemented | X |
| Upon final stabilization, all temporary ESPC BMPs will be removed | X |
| Sediment will be removed from silt fence when it accumulates to one half the height of the fence |  |
| Sediment will be removed from silt sock when it accumulates to one half the height of the silt sock |  |
| Sediment will be removed from straw bales when it accumulates to one third the height of the straw bale |  |
| Rock construction entrances will be maintained to adequate stone depth |  |
| Rock construction entrances will be cleaned out if clogged |  |
| Sediment will be removed from rock filters when it reaches one half the filter height |  |
| All accumulated sediment removed from any practice will be disposed of properly |  |

| 7. EROSION CONTROL PLAN DRAWING |  |  |  |
| --- | --- | --- | --- |
| A plan or drawing illustrating your ESPC plan is required. Section one is a blank sheet which may be used for this purpose. If you have a copy of a document showing your site, you may use that as well. Ensure the following, if applicable, are shown on the illustration in sufficient detail. |  |  |  |
| Name of project |  | Location and type of ESPC practice |  |
| North arrow |  | Property lines |  |
| Scale |  | Legend or key |  |
| Streams or ponds (include name if known) |  |  |  |
| Existing features such as buildings or roads |  |  |  |
| Proposed features |  |  |  |
| Direction of slopes |  |  |  |
| Soil types |  |  |  |

SECTION 1

ESPC PLAN DRAWING

|  |  | 540 | 538 | 2 foot contour lines |  | 536 | 534 |  | SAMPLE ESPC PLAN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | ClC2 ClB2 |  | 532 |  |  |  |  |
|  |  |  |  | DIVERSION CHANNEL (IF NEEDED) | PROPOSED BURIAL AREA | SOIL STOCKPILE |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | BIRD RUN |  | PROJECT NAME: CHICKEN BURIAL PIT_________ MAP KEY Property Line LIMIT OF DISTURBANCE CONSTRUCTION ENTRANCE FABRIC FILTER FENCE ROCK FILTER SWALE OUTLET TEMPORAY SWALE STREAM SOIL TYPE SCALE: 1 INCH = 40 FEET |  |  |
|  |  |  |  |  |  |  |  |  | CHICKEN HOUSE ROAD |  |  |  |  |  |  |

|  |  |  |  |  |  |  |  |  |  |  |  |  | PROJECT NAME: ______________________________________ MAP KEY SCALE: 1 INCH = ___________ FEET |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SECTION 2

COMMON ESPC BMPs

This section describes several common BMPs that are typically used on small project sites.   This section will describe each BMP, its advantages and limitations, provide installation and maintenance information and provide diagrams of the BMP where applicable.  These BMPs are listed below.

  - Vegetative stabilization

  - Sediment barriers such as filter fabric fence, filter socks and straw bales

  - Rock construction entrance

  - Swales

  - Rock filters

  - Rock outlet protection

  - Sediment trap

  - Inlet Filter Bag

These BMPs have been selected for use in this guide because they are:

- typically at the lower cost end of BMPs

- typically require minimal installation effort

- typically well suited for controlling runoff and sediment from small sites

It should be kept in mind that there are other BMPs that can be used.  For example, large sediment basins are often used on larger projects.   However, the intent of this guide is to provide simple and easy to understand guidance for small projects where the BMPs covered in this section should provide adequate erosion and sediment control.   Where more complicated and larger BMPs are needed, this guide is not applicable.

Vegetative stabilization

Vegetated stabilization is the use of vegetation to stabilize disturbed areas.  This includes areas where earth disturbance is final or temporarily completed.  Vegetation is commonly some type of grass that is planted on areas that have been disturbed.  The intent is to provide vegetative cover for disturbed areas as soon as possible after disturbance to protect exposed soil and reduce the potential for erosion of the soil.

AdvantageS:

-Relatively inexpensive

-Relatively simple

-Applicable on most situations

LimitationS:

-May be difficult to establish vegetation

during certain times of the year

-Soil amendments may be needed where

An example of vegetated stabilization - Vegetation established on this slope prevents erosion.  Courtesy of Dauphin County Conservation District

existing ground is not suitable for growth

Installation:

-Grass should be planted as soon as possible after disturbance is permanently or temporarily completed

-Topsoil should have a minimum depth of 4 to 8 inches

-Compacted topsoil should be loosened to a depth of 6-12 inches before seeding

-If topsoil needs to be added, the area should be loosened to a depth of 3-5 inches before applying topsoil

-If soil amendments are needed add amendments as indicated by a soil test or the Penn State Agronomy Guide.

-Seed mixtures should be selected based on the site factors such as:

amount of sunlight                    soil pH                    moisture                    slope                    expected use

-The seeded area should be mulched immediately after seeding

-On slopes of 8% or more mulch should be held in place with appropriate netting

-Erosion control blankets should be used where the site is close to surface water

There are a wide variety of methods for seeding and a wide variety of mulches, netting and control blankets available.  For projects where this guide is applicable, it is likely that simple seeding and mulching will be sufficient.  You may contact the Conservation District for more information.

SEDIMENT BARRIERS

Sediment barriers are typically installed at the perimeter of a site to prevent sediment from leaving the site.  These kinds of BMPs are referred to as perimeter controls.  There are many types of these controls available.  Three common types are discussed here.

FABRIC FENCE

Also called silt fence, is typically used to control sediment

from sheet flow on small project sites.  There are different

heights available for use, depending on site conditions.

Advantages:

-Relatively inexpensive

-Relatively simple

-Applicable on most situations

Limitations:

-Cannot be used where flow is concentrated

-There are limitations on maximum slope length

An example of filter fabric fence – This is a

30 inch reinforced fence.

From Erosion and Sediment Pollution Control program Manual, PADEP, March 2012

Courtesy of York County Conservation District                                                                           .

above the fence.  See slope/slope length Table below

and chart  in section 6.

-Cannot be used in areas where the fence cannot

be completely anchored

-Cannot be used in uncompacted fills or loose soils

MAXIMUM SLOPE LENGTHS ABOVE FILTER FENCE BY TYPE

From Erosion and Sediment Pollution Control program Manual, PADEP, March 2012

| MAXIMUM SLOPE LENGTH, IN FEET ABOVE FENCE (see chart in Section 6 for greater detail) |  |  |  |
| --- | --- | --- | --- |
| Slope Percent | Standard (18” High) Silt Fence | Reinforced (30” High) Silt Fence | Super Silt Fence |
| 2 (or less) | 150 | 500 | 1000 |
| 5 | 100 | 250 | 550 |
| 10 | 50 | 150 | 325 |
| 15 | 35 | 100 | 215 |
| 20 | 25 | 70 | 175 |
| 25 | 20 | 55 | 135 |
| 30 | 15 | 45 | 100 |
| 35 | 15 | 40 | 85 |
| 40 | 15 | 35 | 75 |
| 45 | 10 | 30 | 60 |
| 50 | 10 | 25 | 50 |

Installation:

-Install perpendicular to slope on level grade to efficiently intercept surface flow

-Extend ends at least 8 feet upslope at 45 degree angles to prevent runoff from washing around the ends

-Fabric should be anchored firmly in a trench at least 6 inches deep to prevent underflow

-Backfill and compact the trench

-The ends of the fence should be wrapped around the end stakes and secured to the stake

-Fabric fence larger than 18 inches will require additional methods to secure the fence

Maintenance:

-Fence should be inspected on a regular basis and after each storm

-Remove accumulated sediment when the sediment reaches half the fence height

-If fence is undermined or topped, it should be repaired immediately.  This may also indicate the need for

other erosion control measures

STANDARD FILTER FABRIC FENCE INSTALLATION DIAGRAM

Filter fabric fence installation diagram – From Erosion and Sediment Pollution Control Program Manual, Pennsylvania Department of Environmental Protection, March 2012.

REINFORCED FILTER FABRIC FENCE INSTALLATION DIAGRAM

Reinforced filter fabric fence installation diagram – From Erosion and Sediment Pollution Control Program Manual, Pennsylvania Department of Environmental Protection, March 2012.

FILTER SOCK

A filter sock is similar to a fabric fence in that it is a perimeter control that acts as a barrier to sediment laden runoff from the site.  However, the sock is a tube filled with a compost material.  The filter sock has a slightly greater range of applicability and provides better pollutant removal.

An example of filter sock - From   Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

Courtesy of York County Conservation District

Advantages:

-Relatively inexpensive

-Relatively simple

-Applicable on most situations

Limitations:

-Cannot be used where flow is concentrated

-There are limitations on maximum slope length above the fence. See slope/slope length chart in section 6.

-Cannot be used in areas where the sock cannot be completely anchored

-Cannot be used in uncompacted fills or loose soils

Installation:

-Install perpendicular to slope on level grade to efficiently intercept surface flow

-Extend ends at least 8 feet upslope at 45 degree angles to prevent runoff from washing around the ends

-Remove heavy vegetation and large rock to ensure the sock contacts the ground to prevent runoff from

flowing under the sock

-Secure the sock by driving stakes through the sock or immediately downslope of the sock

FLOW DIRECTION

Filter Sock Installation Diagram -

From Erosion and Sediment Pollution Control Program Manual,  PADEP, March 2012

Maintenance:

-The sock should be inspected on a regular basis and after each storm

-Remove accumulated sediment when the sediment reaches half the sock height

-If the sock is damaged it must be repaired immediately

STRAW BALES

Another type of perimeter control is straw bales.

Straw bales can be an inexpensive way to control

of runoff in the form of sheet flow.

Advantages:

-Relatively inexpensive

-Easy to install

Limitations:

-Maximum lifespan of straw bales is three months

An example of straw bales – Note the sediment

accumulation upslope of the bales

From Erosion and Sediment Pollution Control

Program Manual, PADEP, March 2012

Courtesy of York County Conservation District

-Applicable on very small sites only.

-Undercutting is common if not installed correctly

-Use of straw bales is limited based on the slope and

length of slope above the bales.  See slope/slope

length Table below and chart in section 6.

| MAXIMUM SLOPE LENGTH FOR USE OF STRAW BALES (see chart in Section 6 for greater detail) From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012 |  |
| --- | --- |
| SLOPE PERCENT | MAXIMUM SLOPE LENGTH ABOVE BALES |
| <2 | 150 |
| 5 | 100 |
| 10 | 50 |
| 15 | 35 |
| 20 | 25 |
| 25 | 20 |
| 30 | 15 |
| 35 | 15 |
| 40 | 15 |
| 45 | 10 |
| 50 | 10 |
| >50 | NOT PERMITTED |

INSTALLATION:

-Install on level grade

-The bottom of the bale should be at least four inches below grade to prevent undercutting

-Butt bales tightly together

-Install 2 stakes per bale.  The first stake should be angled toward the adjacent bale to force bales together

-Extend bales at least 8 feet upslope at 45 degree angles to prevent runoff from washing around the ends

Straw Bale Installation Diagram

From Erosion and Sediment Pollution Control Program Manual,  PADEP, March 2012

1. EXCAVATE TRENCH                                               2. PLACE AND STAKE BALES

3. WEDGE STRAW IN JOINTS                                   4. BACKFILL AND COMPACT SOIL

Installation of Straw Bales

From Erosion and Sediment Pollution Control Program Manual,  PADEP, March 2012

MAINTENANCE:

-Bales should be inspected on a regular basis and after each storm

-Remove accumulated sediment when the sediment reaches one third of the above ground bale height

-Undercut or overtopped bales may indicate the need for a different BMP

ROCK CONSTRUCTION ENTRANCE

When vehicles and equipment leave construction

sites, sediment is often carried with them on tires.

This sediment then washes into storm sewers or

road side ditches where it is ultimately delivered to

streams.   A rock construction entrance helps to

prevent the sediment from leaving the site by

removing sediment from tires.

An example of rock construction entrance - From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

Courtesy Lake County Stormwater Management Department, Ohio

ADVANTAGES:

-Relatively inexpensive BMP for vehicular

access to the site

-Effective at preventing most sediment

transport to adjacent roads or streets

LIMITATIONS:

-Not 100 percent effective.  Some sediment will still be transported from the site to roadways

-Must be maintained frequently

-Rock construction entrances are not used to remove sediment from runoff from areas above the entrance

INSTALLATION:

-Topsoil should be removed prior to installing the rock construction entrance

-Place geotextile over existing ground

-Rock should cover the full width of the entrance.  Twenty feet is the recommended minimum width

-If a culvert or pipe is needed to convey clean water, a berm is recommended to prevent crushing the pipe

Rock Construction Entrance Diagram

From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

MAINTENANCE:

-Entrance must be maintained to provide the specified depth of rock by adding rock when needed

-A stockpile of rock should be maintained on site for maintenance

-If the entrance becomes completely clogged, the entrance may need to be reinstalled

-Sediment that is deposited on roadways should be returned to the construction site

-Do not wash or sweep sediment on roadways, into gutters, ditches, inlets or other areas where it can be

washed into streams or storm sewers

SWALES

Swales are conveyance channels that collect and convey runoff.   For small sites, the typical use may be to capture and convey upslope runoff around disturbed areas.   Swales are also used to convey runoff from disturbed areas to sediment removal BMPs such as a sediment basin.  Swales can be lined with either vegetation or rock.

VEGETATED SWALE - Swales can intercept runoff and carry it safely around a project site.   Courtesy of Dauphin County Conservation District.

ROCK LINED SWALE - Swales carry runoff safely down

a steep slope.   From Erosion and Sediment Pollution

Control Program Manual, PADEP, March 2012

ADVANTAGES:

-Swales can prevent upslope runoff from reaching disturbed areas.   This reduces erosion and sediment transport.

-Swales may reduce or eliminate the need for additional controls

LIMITATIONS:

-Swale outlets may need additional stabilization

-Swales may not be practical on some sites

-Swale design may be complex

INSTALLATION:

Installation of swales is dependent on the design.  Consult the DEP publication “Erosion and Sediment Pollution Control Program Manual,  March 2012” for more information.  General installation includes:

- swales should be stabilized immediately after  installation

- Unless the sod is to be installed, a fabric will be needed to stabilize the swale until the grass is established

MAINTENANCE:

-Inspect swales frequently

-Damage to swales should be immediately repaired

-Grass should be maintained at two to three inches

-Swales should be cleaned out if channel depth is reduced by 25%

ROCK FILTERS

Rock filters are rock barriers typically used in swales or channels to slow runoff.  This reduces the erosion potential of the flow and allows sediment to drop out.

ADVANTAGES:

-Rock filters can be used with swales to increase

effectiveness

-Relatively inexpensive and easy to construct

LIMITATIONS:

-Rock filters are not a substitute for

An example of a rock filter - Rock filters, such as the one above can be used to increase efficiency in swales.  From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

Courtesy of York County Conservation District

appropriate channel linings

-Rock filters may not be used where the channel

depth is less than two feet

INSTALLATION:

-The filter should be one half the channel depth

-Use R-3 rock for channels up to 3 feet deep

-Use R-4 rock for channels over 3 feet deep

-Construct the filter with a 6 inch depression in the center

-Place a 1 foot thick layer of small rock (AASHTO #57 or smaller) on the upstream side of the filter

Rock Filter Diagram

From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

MAINTENANCE:

-Inspect filters weekly and after each rain

-Repair filters immediately if needed

-If the filter is clogged, immediately replace the filter

-Sediment should be removed when it reaches one half the filter height

ROCK OUTLET PROTECTION

Rock outlet protection, also called rip-rap, is

used to prevent erosion where channels or

pipes discharge.

ADVANTAGES:

-Rock outlet protection is easy to install

-Rock outlet protection is relatively inexpensive

LIMITATIONS:

-Discharge velocity may not exceed 17 feet

An example of rock outlet protection - From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

per second

-Sufficient area must be available for installation

-Must be installed on level grade

INSTALLATION:

-The diagram to the left is a general construction

Diagram.  The dimensions and rock size must be

determined by calculating the discharge velocity.

If not designed and installed correctly, it is likely

that erosion will result.  Consult the   Erosion and

Sediment Pollution Control Program Manual,

PADEP,  March 2012 or contact the Conservation

District for guidance.

ROCK OUTLET PROTECTION DIAGRAM - From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012

MAINTENANCE:

-Inspect the outlet protection weekly and after each runoff event

-Replace displaced stone immediately

-Look for evidence of erosion around the structure

SEDIMENT TRAP

A sediment trap is a small ponding area which

collects runoff from a disturbed area.  Sediment

in the collected runoff settles in the trap.

ADVANTAGES:

-Sediment traps are not dependent on slope

-Sediment traps can control areas up to five acres

-Sediment traps are less expensive than detention

basins

SEDIMENT TRAP - This sediment trap collects runoff from the disturbed site.   Note the baffling to increase flow length.   From Erosion and Sediment Pollution Control Program Manual, PADEP, March 2012 Courtesy of York County Conservation District

LIMITATIONS:

-Depending on drainage area, sediment traps

may need to be relatively large

-sediment traps should not be located on unstable

soils or steep slopes

INSTALLATION:

-Install sediment traps at the downslope most point of the site

-Discharge to a stable, erosion resistant area

-A rock berm may be used for an outlet structure

-The trap must have a minimum of 2,000 cubic feet of storage for each acre contributing runoff to the trap

-Areas with fine textured soils will require 5,300 square feet of surface area for each acre contributing runoff

to the trap

-Minimum Flow lengths are 4 to 1 in special protection watersheds and 2 to 1 in other watersheds

-The minimum storage depth is three feet; one foot for sediment storage, one foot for runoff storage and one

foot of freeboard

-The maximum embankment height is five feet with maximum slopes of 2H:1V

-Do not use fill material with roots, organic material, large stones or woody vegetation

-Compact the embankment in lifts of 9 inches or less

-Seed and mulch or provide other stabilization upon completion

-Trap should discharge outflow safely to receiving waters without causing erosion

MAINTENANCE:

-Access should be provided for sediment removal and other maintenance activities

-Inspect traps weekly and after each rain

-Replace displaced rock from the outlet structure immediately

-Any damage, erosion or clogging of the components of the trap should be immediately repaired

-Remove sediment from the trap when the sediment storage area is full.  A cleanout stake placed in the

middle of the trap can be used to determine when to remove sediment

TYPICAL DESIGN ELEMENTS OF A SEDIMENT TRAP

CROSS SECTION

V

H

MAXIMUM                                             MINIMUM EMBANKMENT

EMBANKMENT                                      WIDTH 5 FEET

MINIMUM 1 FOOT OF FREEBOARD

MINIMUM 1 FOOT OF RUNOFF STORAGE

MINIMUM 1 FOOT OF SEDIMENT STORAGE

SLOPE IS 2H:1V

MAXIMUM EMBANKMENT

HEIGHT 5 FEET

ROCK OUTLET – EXTENDS TO BOTTOM OF TRAP WITH FILTER FABRIC COVERING ROCK TO TOP OF SEDIMENT STORAGE.

____________________________________________________________________________________

IN FLOW                                                              PLAN VIEW                                                                                                                IN FLOW

MINIMUM FLOW LENGTH 10 FEET

MINIMUM FLOW LENGTH TO WIDTH RATIO 4:1 IN HQ AND

EV WATERSHEDS, 2:1 IN OTHER WATERSHEDS

MINIMUM STORAGE 2,000 CUBIC FEET

PER ACRE DRAINING TO TRAP

ROCK OUTLET

R-3 ROCK LARGER WITH AASHTO #57 OR SMALLER ON INSIDE

______________________________________________________________________________________

THE ABOVE DIAGRAMS CONTAINS TYPICAL ELEMENTS OF A SEDIMENT TRAP FOR USE ON A SMALL PROJECT SITE.  FOR FURTHER GUIDANCE ON DESIGN AND INSTALLATION CONTACT THE CONSERVATION DISTRICT.

INLET FILTER BAG

Inlet filter bags can be used to prevent

Sediment from entering storm sewer

Systems which discharge to surface

waters such as streams or lakes.  If

the discharge is to a sediment basin or

sediment trap, filter bags are not needed.

ADVANTAGES:

-Relatively inexpensive

-Easy to maintain

An example of an Inlet Filter BAG - The edges of the bag can be seen around the grate. From Erosion and Sediment Pollution Control Program Manual,   PADEP, March 2012

Courtesy Northampton County Conservation District

-Effective when maintained

LIMITATIONS:

-Should not be used on roads where

possible ponding could create a traffic

hazard

-Maximum drainage area is one half acre

-Not recommended for use at edge of fill slopes

INSTALLATION:

-Fabric tensile strength must be a minimum of 120 pounds

-Burst strength must be a minimum of 200 pounds

-Tear strength must be a minimum of 50 pounds

-Bag must be capable of catching all particles not passing a number 40 sieve (0.4 millimeters)

-Filter bags that fit over the inlet grate are not recommended

-Install according to manufacturer’s specifications

-Maintain extra bags on site for replacement

MAINTENANCE:

-Inspect weekly and after each rainfall

-Clean or replace the bag when half full

-Dispose of accumulated sediment properly

-Clean or replace the bag if clogged (indicated by ponding or runoff bypassing inlet)

-Replace torn or damaged bags immediately

-If bag is to be reused, rinse to a sediment trap or basin

Installation of Inlet FilTer Bag for a Type C inlet - Note the blocking of the upper inlet in the Installation Detail to prevent sediment laden runoff from entering the inlet unfiltered.   From Erosion and Sediment Pollution Control Program Manual,   PADEP, March 2012

__________________________________________________________________________________________

Installation of Inlet FilTer Bag for a Type M inlet -   From Erosion and Sediment Pollution Control Program Manual,   PADEP, March 2012

SECTION 3

RESOURCES

LITERATURE

Erosion and Sediment Pollution Control Program Manual, Pennsylvania EP, March 2012.

Erosion Control and Conservation Plantings on Noncropland, Pennsylvania State University, 1997

The Agronomy Guide, 2013-2014, The Pennsylvania State University, 2013

WEBSITES AND RESOURCES

DEP – www.depweb.state.pa.us

NRCS Soil Survey –  http://websoilsurvey.nrcs.usda.gov/app/

Penn State Publications – http://pubs.cas.psu.edu/Publications.asp

Penn State Agronomy Guide – http://pubs.cas.psu.edu/FreePubs/PDFs/agrs026.pdf

Erosion Control and Conservation Plantings on Noncropland - http://www.dauphincd.org/erosion/EROSION%20AND%20SEDIMENT%20CONTROL%20ON%20NONCROPLAND.pdf